VHDL Cheat Sheet – Combinational Logic

For a complete reference, go to http://www.acc-eda.com/vhdlref/

# Libraries

- ALWAYS start each VHDL file with the following 2 lines

library ieee;

use ieee.std_logic_1164.all;

# Data Types

- Two most common: std_logic, std_logic_vector(<N> downto 0)

# Entities

- Defines a black-box interface for your module

entity MyEntity is

port ( a, b : in std_logic;

c : out std_logic);

end entity;

# Architectures

- Defines your implementation of your entity

- Include any signal declarations before ‘begin’

architecture main of MyEntity is

signal <blah>;

begin

<blah>

end architecture;

# Component Declaration

- Any architectures that use an external component must have a component declaration

architecture main of MyEntity is

component DFF

port(	d : in std_logic;

q : out std_logic);

end component;

signal EntityD, EntityQ : std_logic;

begin

DFF_1 : DFF

Port map (d => EntityD, q => EntityQ);

end architecture;

# Concurrent Assignment

- Describes a wired connection

a <= b and c;

The following 2 can only be written OUTSIDE a process

# Selected Assignment

With Sel select

Y <=	A when “00”,

B when “01”,

C when “10”,

D when others;

# Conditional Assignment

Y <= A when Sel = “00” else

B when Sel = “01” else

C when Sel = “10” else

D when others;

# Combinational Processes

- Another way to describe combinational logic

- A process is combinational if it doesn’t infer any registers

process(<sensitivity list>)

begin

<blah>

end process;

The following 2 can only be written INSIDE a process

# If-Then-Else

process(a)

begin

if a = ‘0’ then

q <= ‘0’

else

q <= ‘1’

end if;

end process;

# Case Statement

process(a)

begin

case a

when ‘0’ => q <= ‘0’;

when others => q <= ‘1’;

end case;

end process;

# Sequential Processes

- Describes sequential logic

- A process is sequential if it infers at least one register

process(clk)

begin

if rising_edge(clk) then

<blah>

end if;

end process;